# FPU

# Nomenclature

Generally, the file name reflects the name of the module.

Binary floating-point modules begin with the prefix fp<name> to distinguish the modules from other types and formats so that modules for different types may be used in the same project.

For example, fpMultiply32nr.sv is a binary floating-point 32-bit precision multiply that has a normalized and rounded output.

The module names are suffixed with the precision of the operation supported. A number is used as there may be odd sized modules.

fpCompare64.sv for instance is a binary floating-point 64-bit double precision comparator.

Modules containing the ‘nr’ as part of the suffix are normalized and rounded outputs. Without the ‘nr’ the module output is a raw extra wide output.

Modules may also be suffixed with the latency of the module in clock cycles, as in fpNormalize32L2.sv which is a normalizer with a latency of two.

Strictly combinational logic modules have the suffix ‘combo’ in the module name as in: mult16x16combo.sv

# Pipelining

Most of the modules are fully pipelined and may start a new operation every clock cycle. Exceptions include complex operations like division and square root.

There are pure combination modules to allow the synthesizer to optimize the timing.

# Types

| Name |  |  |
| --- | --- | --- |
| FP64 | 64-bit binary floating-point format, used for first input and final output. This is an IEEE compatible format. |  |
| FP64X | 64-bit expanded binary floating-point format suitable for normalization. It includes a double-width significand. |  |
| FP64N | 64-bit output of the normalizer, to be fed into a rounding module. |  |

# Summary of Modules

## 64-bit Double Precision

| Name | Latency | Output Type |  |
| --- | --- | --- | --- |
| f2i64.sv | 1 |  | Converts 64-bit binary floating point value to an integer. |
| fp64Pkg.sv |  |  | This file is to be imported to support 64-bit binary floating-point |
| fpAddsub64.sv | 13 | FP64X | 64-bit binary floating-point adder / subtractor. Has expanded representation for input to normalizing and rounding. |
| fpAddsub64nr.sv | 24 | FP64 | 64-bit binary floating-point adder / subtractor with normalized and rounded output. |
| fpNormalize64.sv | 8 | FP64N | 64-bit normalization module that accepts an expanded input value FP64X and outputs a value suitable for rounding – FP64N. |
| fpRound64.sv | 3 |  | 64-bit rounding module that accepts the output of the normalizer (FP64N) and outputs a standard binary floating-point value (FP64). |
| fpCompare64.sv | 0 | - | 64-bit binary floating-point comparator that outputs a 16-bit bus with bits representing different relationships of the input |
| fpCordic.sv | >30 |  | Cordic module to used to compute trig functions like atan, sin and cos. |
| fpCvt8To64.sv | 0 | FP64 | Converts an 8-bit binary float to a 64-bit binary float. |
| fpCvt16To64.sv | 0 | FP64 | Converts an 16-bit binary float to a 64-bit binary float. |
| fpCvt32To64.sv | 0 | FP64 | Converts an 32-bit binary float to a 64-bit binary float. |
| fpDecomp64Reg.sv | 0/1 |  | Decomposes a 64-bit binary float into its constituent parts, including zero, infinity and NaN detection. This file contains both unregistered and registered output versions. |
| fpDivide64.sv |  |  | 64-bit binary floating-point divider with option to use standard division with a radix of 16 (default) or the GoldSchmidt algorithm |
| fpdivr16.sv |  |  | Radix 16 divide component used in the fpDivide64 module |
| fpFDP64L5.sv | 5 | FP64X | 64-bit binary-floating point fused dot product. |
| fpFMA64combo.sv | 0 | FP64X | 64-bit binary floating-point fused multiply-add module. Combinational logic only. |
| fpMultiply64.sv |  | FP64X | 64-bit binary floating-point multiplier. |
| fpMultiply64nr.sv |  | FP64 | 64-bit binary floating-point multiplier with normalization and rounding. |
| fpNextAfter64.sv | 2 | FP64 | Computes the next representable 64-bit value |
| fpNormalize64.sv | 8 | FP64N | Normalizes the intermediate format for use with rounding. |
| fpNormalize64combo.sv | 0 | FP64N | Pure combinational logic normalization |
| fpNormalize64L2.sv | 2 | FP64N | Normalizes the intermediate format for use with rounding. |
| fpRes64.sv |  | FP64 | Reciprocal estimate. |
| fpRound64.sv | 3 | FP64 | Rounds the output of a normalizer, 64-bit binary floating point output FP64. |
| fpRound64L1.sv | 1 | FP64 | Rounds the output of a normalizer, 64-bit binary floating point output FP64. |
| fpRound64combo.sv | 0 | FP64 | Combinational version of the rounding component. |
| fpScaleb64.sv | 2 | FP64 | Performs the Scaleb operation adding the second operand to the exponent. |
| fpSigmoid64.sv | 3 | FP64 | Sigmoid approximation |
| fpSign64.sv | 0 | FP64 | Returns the sign of the number as +1 or -1. |
| fpSincos64.v |  | FP64 | Computes the sin and cosine of an angle which must have been previously reduced. Uses the fpCordic module |
| fpSqrt64.sv |  | FP64 | Computes the square root of a number. Uses the standard algorithm. |
| fpTrunc64.sv | 1 | FP64 | Truncates the fractional part of a binary floating-point number. |
| i2f64.sv | 1 |  | Converts a 64-bit integer to a binary floating-point number. The number may be rounded. |

## Supporting Modules – Precision Independent

| isrqt.sv | Calculates the integer square root using the standard algorithm |
| --- | --- |
| mult16x16.sv | 16 x 16 bit multiplier, single clock cycle |
| mult16x16combo.sv | 16 x 16 bit combinational logic multiplier |
| mult32x32.sv | 32x32 bit multiplier, Karatsuba method |
| mult32x32comb.sv | 32x32 bit combinational logic multiplier |
| mult64x64.sv | 64x64 bit multiplier, Karatsuba method |
| mult64x64comb.sv | 64x64 bit combinational logic multiplier |
| redorN.sv | reducing OR function |